NUMERICAL MODELLING OF THE HYDRODYNAMICS, HEAT TRANSPORT AND

EVAPORATION OF ALUMINA NANOPARTICLES-BIODIESEL SURROGATE

DROPLET

BY

OLAMIDE, OLALEKAN OLAOLUWA

B.Sc. (OOU, IBOGUN), M.Sc. (UNILAG, LAGOS)

A Thesis submitted to the Department of Mechanical Engineering, College of Engineering,

Federal University of Agriculture, Abeokuta in partial fulfillment of the requirements for

the award of Doctor of Philosophy in Thermofluids

October, 2023

ABSTRACT

Biodiesel is a renewable fuel that can readily replace petrodiesel in internal combustion

engines and furnaces. However, it contains less energy density than petrodiesel, which can

be enhanced by adding energetic nanoparticles. This study modelled the flow, heat transfer

and evaporation characteristics of an isolated biodiesel surrogate (methyl decanoate)

droplet containing Al 2 O 3 nanoparticles sedimenting in air. The effects of nanoparticle

volume fraction (φ), Reynolds number (Re) and evaporating temperatures in the range 0 to

0.1; 0.1 to 250; and 523 to 723 K, on flow vigour, Nusselt number (Nu), Sherwood number

(Sh) and droplet regression, respectively, were investigated. The influence of internal

circulation on the modes of heat transfer during non-evaporative heating was also

examined with the initial droplet and ambient temperatures of 300 and 400 K, respectively.

The problem governing equations, including the continuity, momentum and energy, were

discretized and solved using the finite volume method with ANSYS Fluent 18.1 while a

MATLAB program was written for implementing the evaporation model. At the domain

inlet, outlet, walls and centreline, the Dirichlet, pressure outlet, Neumann and axisymmetric

boundary conditions were imposed, respectively. User-defined functions were written in

C++ to prescribe the continuity of tangential velocity and shear stress at the liquid-gas

interface. A mesh consisting of 151423 nodes was chosen for the simulation after

conducting a grid independence test. The validations of the drag, heat transfer and

evaporation rates were in good agreement (±10%) with the experimental and numerical

data obtained from literature for Re up to 100. The droplet&#39;s internal flow structure was

similar to the Hill’s vortex for all Reynolds numbers. At critical Reynolds number, Re =

23.29, lung-shaped vortices were formed behind the droplet and grew in size with the increase in Re. At Re of 0.1, the isotherms inside the droplet were concentric about its

centre, signifying pure diffusion. The isotherms within the droplet transformed from

concentric circles at low Re to two deformed cells at high Re. There was an increase in Nu

by 8.56 and 110.64%; 15.96 and 41.78% when Re increased from 0.1 to 50 and 250; and φ

from 0 to 0.02 and 0.1, respectively. The square of the droplet diameter regressed linearly

at a faster rate with the increase in Re than φ, obeying the classical D-squared law during

evaporation for all the cases considered. An increase in φ from 0 to 0.04 enhanced the heat

transfer during evaporation by 0.24 and 0.30% for Re of 100 and 200, respectively.

Sherwood numbers increased with increasing φ for non-isothermal droplet evaporation but

did not surpass 0.1%. The reduction in evaporation time for φ of 0.04 and 0.1 at Re of 100

were 1.66 and 1.20% respectively. Heat transfer enhancements were observed with the

addition of Al 2 O 3 nanoparticles in methyl decanoate, while the changes in flow and mass

transfer characteristics were marginal. The modelling and simulation of the evaporation

characteristics of an isolated Al 2 O 3 nanoparticles-biodiesel surrogate droplet showed

enhanced performance.